CERTIFICATE OF QUALITY

'TO WHOM IT MAY CONCERN'

PAGE 1 OF 2

REF:  Certificate No. COA-2026-001

DATE:  11 May 2026

DESCRIPTION OF GOODS

NAME OF COMMODITY                 :     Bauxite

QUANTITY                          :     200000 METRIC TONS

COUNTRY OF ORIGIN                 :     Guinea

PACKING                           :     in Bulk

NAME OF THE CARRYING VESSEL       :     MV Bullfrog

PORT OF LOADING                   :     Conakry

PORT OF DISCHARGE                 :     China

B/L NO. & DATE                    :     01 & DATED 10.05.2026

=========================================================================

In accordance with the instructions received from the shipper, we attended at, Conakry during the period 01.05.2026 to 10.05.2026 for the purpose of drawing representative samples of the consignment of Bauxite while the cargo was being loaded on board the vessel MV Bullfrog at Conakry. We certify as under:

AT Conakry       :     Cargo loading commenced on  01.05.2026

Cargo loading completed on  10.05.2026

SAMPLING PROCEDURE

Systematic mass based sampling carried out in accordance with BIS 1405 throughout course of loading. Sublotwise gross sample was constituted by collecting requisite number of sample increments while loading the cargo into the vessel at Conakry. Individual, Sublotwise gross samples were subject to size analysis and further processed to obtain for moisture determination and chemical analysis.

Sub-lot wise samples drawn as above mixed together to prepare composite sample representing the entire shipment.

PAGE 2 OF 2

REF:  Certificate No. COA-2026-001

DATE:  11 May 2026

Composite sample representing the shipment at Conakry was divided into 3 parts and sealed with our monogram. After retaining 2 parts for future reference, one part tested in our lab with the following results.

The analysis as per IS 1493 for the specifications computed for the cargo shipped are as under:

SPECIFICATIONS:

THE ACTUAL RESULT OF THE TEST FOR CHEMICAL COMPOSITIONS (ON DRY BASIS)

(Percentage by weight)

Al2O3: 45%

Sio2: 3%

THE ACTUAL RESULT OF THE TEST FOR FREE MOISTURE LOSS AT 105 DEGREES

CENTIGRADE           :  5%   PCT

THE ACTUAL RESULT OF THE TEST FOR PHYSICAL SIZES  (ON NATURAL BASIS) :

SIZE :

Above 10MM: 5%

This Certificate reflects our findings at the time and place of inspection only and does not refer to and any other matter.

FOR SGS

AUTHORIZED SIGNATORY

ISSUED AT LOADING PORT

This certificate contains only two pages.

DISCLAIMER: The information contained in this document may be privileged and/or confidential, and is intended only for the use of the person to whom it is addressed. If the reader of this message is not the intended recipient (or such recipient's employee or agent), you are hereby notified not to read, distribute, use or copy this document or the materials attached. If you have received this document in error, please notify the sender and delete the document from your computer. In compliance with regulatory requirements, all messages sent to or from this server are archived and may be read by someone other than the recipient. This document has been scanned for viruses and malware, and may have been automatically archived by Bullfrog Group.

Issued by Bullex Trading Platform